JALRAKSHAK — LIMITATIONS, SCOPE & RED-TEAM REPORT

JalRakshak SIH'26 — Evidence Hub

# Critical limitations

| Area | Limitation | Impact | Safe wording |
| --- | --- | --- | --- |
| Terrain | GLO-30 is DSM. | Surface objects can affect elevations. | Say DSM; expose resolution/limitations. |
| Vertical datum | Not established in all records. | Potential systematic elevation issue. | Mark NOT_ESTABLISHED until verified. |
| Calibration | No established physical Tehri calibration. | Cannot claim calibrated model. | Use demonstration/scenario language. |
| Remote sensing | SAR confounders/event matching. | Not automatic ground truth. | Research-only discrepancy layer. |
| Travel | Static speed assumption. | No live traffic. | Configured assumption. |
| Road impact | Inundation ≠ structural failure. | Avoid structural-damage wording. | Road flood-impact/inundation. |
| SPH/Delft3D | Interfaces only unless runs evidenced. | Not full implementation. | Interface status. |
| Exposure/loss | No complete validated vulnerability chain. | Cannot claim accurate financial loss. | Framework only. |
| Generalization | Independent test worlds only. | Not universal rivers. | Scoped data-driven architecture. |
| Arbitrary HEC-RAS scenario generation | Not universal end-to-end. | Limits flexibility. | Prepared result ingestion. |
| Operational deployment | Prototype evidence is not certification. | Emergency use needs stronger validation. | Demo-ready research prototype. |

# Red-team questions

What exact HEC-RAS artifact produced this map?

Where is the checksum?

How was arrival time calculated?

Why is the road affected?

What happens when an edge has no hydraulic value?

What is the speed assumption?

Can you create a brand-new arbitrary HEC-RAS scenario?

Where is the SPH run?

Where is the Delft3D run?

What is the field-validation dataset?

Does Sentinel-1 prove the model is correct?

Why should an officer trust the departure deadline?