JALRAKSHAK — TEHRI SOURCE & PARAMETER CARD

JalRakshak SIH'26 — Evidence Hub

# Study-area identity

Study area: Tehri Dam / Bhagirathi downstream corridor. Keep dam facts, modeled assumptions, terrain, roads and scenario parameters separate.

# Source-derived context recorded in project materials

| Parameter | Value | Classification | Use |
| --- | --- | --- | --- |
| Dam height | 260.5 m | SOURCE-DERIVED | Context |
| Crest length | 575 m | SOURCE-DERIVED | Context |
| FRL | EL 830 m | SOURCE-DERIVED | Reference where explicitly configured |
| Gross storage | 3540 MCM | SOURCE-DERIVED | Context/input only when stage-storage relationship is documented |
| MWL | EL 835 m | SOURCE-DERIVED | Context/reference |

# Prepared demonstration scenarios

| Scenario | Qp | Peak time | Base | Classification | R02 decision example |
| --- | --- | --- | --- | --- | --- |
| MINIMUM | 28,500 | 1.5 h | 180 min | CONFIGURED / engineering assumption | Arrival T+95:00; deadline T+79:21 |
| CENTRAL | 65,000 | 1.0 h | 180 min | CONFIGURED / engineering assumption | Arrival T+60:00; deadline T+44:21 |
| MAXIMUM | 115,000 | 0.75 h | 180 min | CONFIGURED / engineering assumption | Arrival T+45:00; deadline T+29:21 |

# Critical presentation correction

The T+60:00 arrival / 12:39 travel / 03:00 buffer / T+44:21 departure example is CENTRAL, not MINIMUM. The minimum example is T+95:00 / T+79:21; maximum is T+45:00 / T+29:21.

# Evidence needed for stronger breach claims

Authoritative emergency-action-plan material relevant to downstream breach planning.

Documented breach width, final breach elevation, formation time/progression method.

Reservoir initial condition and stage-storage relationship.

Upstream inflow and downstream boundary condition.

Manning roughness and mesh/terrain justification.

Native HEC-RAS project, geometry, plan, flow/boundary files and HDF5.

Run log, mass/volume accounting and warnings.

Explicit classification for every parameter.