JALRAKSHAK — TERRAIN DATA & 3D CONTEXT REPORT

JalRakshak SIH'26 — Evidence Hub

# Terrain source

The documented terrain pipeline uses Copernicus DEM GLO-30. It is a Digital Surface Model (DSM), not a bare-earth DTM. Buildings, vegetation and infrastructure may influence elevation.

# Required metadata

Dataset ID / tile or product identifier.

Release/acquisition information.

Download date.

CRS and transformation.

Horizontal and vertical units.

Resolution.

Bounds and dimensions.

Nodata value/count.

Checksum.

Licence and attribution.

All clipping, reprojection, resampling and conditioning operations.

# QA checklist

Confirm CRS.

Confirm units.

Check nodata.

Inspect vertical range.

Inspect dam and river corridor.

Inspect gross artifacts.

Document terrain conditioning; never silently alter the terrain.

# Critical limitation

Do not call GLO-30 “99% accurate.” Its DSM nature and any unresolved vertical-datum compatibility must be exposed. If vertical-datum compatibility is not established for the hydraulic setup, mark it NOT_ESTABLISHED.

# 3D visualization requirements

Use real elevation.

Use actual HEC-RAS hydraulic flood extent/depth where shown as hydraulic evidence.

Display dam, river, roads and route from sourced geometry.

Show data-source attribution and resolution/date.

Display vertical exaggeration if used.

Treat 3D as context/evidence, not proof of hydraulic accuracy.