JALRAKSHAK — ROAD–HYDRAULIC COUPLING REPORT

JalRakshak SIH'26 — Evidence Hub

# Why coupling matters

Hydraulic maps describe water behavior. Emergency officers need an interpretation of that behavior on a route: where the route becomes temporally constrained and how much departure time remains under the selected scenario.

# Hardened coupling method

Projected road geometry in EPSG:32644 for the documented Tehri workflow.

Road densification ≤50 m.

Exact perpendicular distance to hydraulic cells.

Strict 150 m baseline corridor for the 100 m mesh.

Old 1200 m nearest-cell method rejected because distant cells could create false inundation.

# Sensitivity record

| Corridor | Coupled cells | Inundated cells | Max depth | Meaning |
| --- | --- | --- | --- | --- |
| 50 m | 75 | 12 | 30.89 m | Narrow sensitivity |
| 100 m | 152 | 22 | 31.63 m | Intermediate |
| 150 m | 230 | 34 | 31.70 m | Baseline |
| 200 m | 307 | 47 | 31.86 m | Broader sensitivity |
| 500 m | — | — | 37.25 m | Distant-cell influence increases |
| 1200 m | — | — | 39.19 m | Rejected; false association observed |

# Travel-time assumption

The demonstration uses a static configured 50 km/h travel-speed assumption. It is not live traffic, dynamic congestion, or a validated evacuation-speed distribution.

# Per-edge evidence

Hydraulic arrival time

Depth where displayed

Velocity where displayed/used

Coverage/nodata state

Source scenario and artifact provenance